Hull

*Designer: Robert Perry;   *LOA: 34’ 10.5”;   *LWL: 29’ 7.5”;   *Beam:  11’ 2”;   *Draft: 5’6”;   *Displacement: 21,140 lbs;   *Ballast: 8,200 lbs (internal iron);   *Sail Area: 716 sq. ft.;   *USCG No.: 624796;   *Port of Registry: Seattle, WA;   *Builder: Ta Shing Yachts, Taiwan;   *Year Built:  1979;   *Construction: Hand-laminated fiberglass with foam core insulated hull;   *Double-ender design with bowsprit and full keel with cutaway forefoot.

Rigging, Winches & Sails

Rigging:  *Cutter rig (staysail and jib head sails), single spreader;  * Rigging: 1/4” and 5/16” stainless steel wire (all standing rigging replaced 1996);   *Aluminum keel-stepped mast (replaced in 1996) with internal halyards for main, jib, staysail and spinnaker;   *Spare external masthead halyard with capability to add a second external halyard;   *Aluminum boom with blocks, jam cleats and winch for managing triple-reefed main;   *ProFurl NC42 jib reefing and furling system;   *Removable staysail stay with quick-release lever enables sailing as a cutter or sloop;   *Running back stays, each with 4-to-1 tackle and cam cleat;   *Schaeffer blocks throughout;  *Adjustable aluminum whisker pole used on mast-mounted track;   *Pinrails for managing halyards;  * Port & starboard deck tracks each have two sets of cars and turning blocks for trimming the jib and cruising spinnaker;   *Port & starboard cabin top tracks have cars and turning blocks for trimming the staysail;   *Mainsheet traveler, cunningham, 3:1 boomvang and outhaul for trimming the mainsail;   *Port & starboard stern snatch blocks enable using jib winches for spinnaker trim.   Winches (10 ea):   *Main halyard: 2-spd Barient 18;   *Jib halyard: 2-spd Barient 18;   *Staysail halyard:  single-speed Barient 16;   *Main outhaul/reefing: single-speed Barlow 16;   *Jib furler:  single-speed Barlow 16;   * Main sheet: 2-spd Barient 18;    *Jib sheets: 2 ea 2-spd self-tailing Lewmar 43’s;   *Staysail sheets: 2 ea single-speed Barlow 10’s.   Winch handles (6 ea):   *8” locking,   *2ea 11” locking,   *8” non-locking,   *2 ea 11” non-locking.   Sails (6 ea):  *Triple-reefed Mainsail;    *Furling Jib with blue acrylic sun strip;   *Furling 130% Genoa;   *Hanked-on Storm Jib;   *Hanked-on Staysail with bag;   *Cruising spinnaker with sock.

Cockpit

*Teak-covered fiberglass coamings surround the cockpit.   *Stern anchor, Lifesling man overboard system, stern tie reel and outboard pads mounted on wrap-around stern pulpit.   *Use Garhauer outboard davit  from cockpit to lift up outboards stored on stern pulpit.   *Stainless steel dinghy davits extend from rear cockpit to store dinghy under way.   *Danforth compass sits on pedestal directly forward of steering wheel and engine controls.  *Garmin chart plotter and Autohelm autopilot are mounted next to steering pedestal.   *Main, jib, staysail and spinnaker sheets all lead to the cockpit for trimming.   *Adjusting mainsheet traveler and furling and unfurling jib is done from the cockpit.  *Anemometer, wind speed, depth sounder and radar can all be viewed from the cockpit.  *Manual bilge pump and ships bell operate from the cockpit.  *Removable cockpit table mounts to teak cup holder on pedestal.  *Cockpit seat and back cushions cover teak seats.

Engine, Fuel & Transmission

Engine: *Perkins 4-108 diesel (completely rebuilt 2008) rated at 50HP;   *Engine hour meter: set to zero at engine rebuild, run approx 25 hours since rebuild;   *Diesel Filter: Racor 500FG;   *Water Witch Model 12V digital engine hour meter with trip reset;   *Stuffing box temperature monitor uses a thermocouple sensor to monitor stuffing box temperature;   Fuel: *Fuel consumption: approx 3/4 gal/hr;   *Fuel tank: epoxy-coated aluminum (installed 1996);   *Fuel capacity: 44.5 gallons in main fuel tank; an additional 20 gallons stored in 4 ea 5-gallon tanks beneath salon floor;   *Fuel tank level: read using “Tank Tender”;   Transmission: *Borg Warner Velvet Drive (rebuilt 2008);   *Fresh-water engine cooling;   *Engine temperature monitor;   *Propeller shaft: 1 ¼” stainless steel   *Propeller: 3-blade bronze;   *Propeller lock: Sarns Proplock prevents propeller from turning while sailing and saves transmission from running without lubrication.

Electrical System

*Batteries:  6 batteries in 3 banks;   *Bank 1:  2 ea Napa 8240 12V marine batteries providing 160 Amp Hrs;   *Bank 2:  2 ea Trojan T125 6V batteries for 12V total providing 240 Amp Hrs;   *Bank 3:  2 ea Trojan T125 6V batteries for 12V total providing 240 Amp Hrs;   *Battery charger:  Xantrex TC40+ multistage 40 amp charger;   *DC voltage and current metering via Quad Cycle Alternator Regulator Controller;   *Inverter: 400W Xantrex XS400 Pure Sine Wave Inverter;   *Inverter Outlets: 3 ea dual-receptacle outlets in Nav Station for laptop, printer, DVD player and LCD TV;  *12V Outlets: 3 ea in Nav Station area;   *Engine alternator upgraded to provide 120 amp capacity;   *Wiring and cabling installed for solar cell;   *Shore Power: 30 amp breaker and one 50’ chord;   *Shore power dual-receptacle outlets (9 total): 1 in quarter berth, 1 in galley, 4 in salon, 2 in V-berth, 1 in bathroom.

Navigation & Communication

Navigation: *Radar: Furuno Model 1800, 9” green phosphor CRT, 3kW nominal peak power, range 24 nm;   *Depth Sounder/Fish Finder: Interphase Probe with forward scanning sonar;   *Cockpit compass: Danforth Constellation Express with 5” dial;   *Portable Silva compass;   *Knotmeter: Swoffer Model 5D, 4” dial analog knotmeter with paddle wheel sensor;   *Anemometer: Swoffer Model 40 wind speed indicator, 4” dial analog meter, anemometer cups and bearing replaced 2010;   *GPS: Garmin 182C color chart plotter with electronic charts from Puget Sound to Glacier Bay, Alaska including the west side of Vancouver Island (updated 2007), cable with NMEA info available at Nav station for input to PC-based apps;   *Autopilot: 2 ea Autohelm 3000 belt-driven systems;   *Running Lights: Aqua Signal Series 40 port, starboard and stern running lights;   *Masthead Lights: Aqua Signal Series 40 Quickfit Tricolor;  *Steaming Light: midway up mast;   *Stainless steel thru-hull bolted mounting plate for windvane.   Communication: *ICOM IC-M402S VHF Marine Transceiver, 25 watt, with built-in DSC capability;   *HF Ground consisting of 100 square feet of copper screen installed under salon floor with connection to terminal at the Nav Station;   *Insulated backstay;   *RG-8 and Tuner Control cables installed from Nav Station to Tuner location in the stern lazarette below the backstay.

Ground Tackle

*Anchor Windlass:  Maxwell 1200HWC, dual-direction heavy duty power windlass with two chain wheels,  handheld remote control on 35’ cable with plug-in in cockpit;   *Starboard anchor and anchor rode: 35lb CQR anchor with 297’ of 5/16” high test galvanized anchor chain;   *Port anchor and anchor rode:  45lb CQR anchor with 207’ of 5/16” high test galvanized anchor chain and 167’ of 9/16” braided nylon;   *Anchor storage in bow:  anchor locker wall divides locker into two sections so chains from different rodes will not tangle;   *Starboard anchor rode has option to feed chain down a tube that leads into the forward V-berth hull to provide better weight distribution in the bow;   *Stern anchor and anchor rode:  22lb high tensile Danforth anchor with 25’ of 5/16” high test galvanized anchor chain and 360’ of 9/16” braided nylon, anchor mounted on stern pulpit, anchor rode stored in canvas bag in stern lazarette;  *Anchor floats: 2 ea with fluorescent green floats and 75’ of braided polypropylene floating line;   *Shore tie line:  550’ of 3/8” braided polypropylene floating line on spool attached to stern pulpit.

Safety

*Bilge pumps: 12V bilge pump installed above water line for easy maintenance, manual bilge pump operates from cockpit;   *Fire extinguishers (3 ea);   *Thru-hulls are bronze with bronze lever-action sea cocks;   *Propane switch for galley stove;   *Lifesling overboard rescue system;   *Flotation cushions (4 ea);   *Life vests (2 ea)   *Emergency flares (3 ea);   *Bronze ships bell;   *Portable spotlight;   *Radar reflector;   *Removable port and starboard stainless steel jack lines running from bow to stern;   *Emergency tiller;   *Protective coaming completely surrounds cockpit;   *Wrap-around double rail bow and stern pulpits;   *Full perimeter stainless steel double lifelines between stern and bow pulpits with port and starboard gates;   *Cockpit pad eyes for safety harness attachment;   *Pedestal guard;   *Handrails on cabin top and dodger;   *Detachable stainless steel boarding ladder;   *Spreader lights;   *Dodger ceiling lamp;  *3-position stainless steel and teak boom gallows.

Dodger & Canvas

Dodger:  *Fiberglass-paneled “Wavestopper” hard dodger with Lexan front and side windows;   *2ea stainless steel tubes on dodger top facilitate solar cell mounting;   *Dodger windows have 2 sets of covers:  yacht acrylic covers for protection and privacy and  “webbed” covers to reduce bright sunlight but still enable viewing;   *Bimini top protects and shades the rear cockpit;    *Full cockpit enclosure:  9 each yacht acrylic panels with Lexan or vinyl windows attach to the bimini top and hard dodger to provide a complete weather-proof enclosure for the cockpit;    *Panels can be removed or added as needed;   *Side and rear panel covers enable storing panels beneath V-berth cushions;   Canvas:  *Acrylic covers protect cabin top hatches, hand-rails, dorade boxes, mast boot and Charlie Noble on cabin roof; acrylic covers also protect bowsprit, port and starboard deck rails, rails surrounding the cockpit, all the winches, the steering wheel, the compass, the GPS and autopilot controls. *Mainsail, jib and staysail covers and bags are all acrylic.

Other Equipment

*Stainless steel dinghy davits provided but not currently installed;  *Garhauer stainless steel outboard lift with 6-to-1 tackle mounts on stern plate;   *Bosun’s chair and 6:1 mast hauling rig with 250’ of 3/8” braided line designed for one person to haul himself up the mast;   *”Dinghy Dangler” heavy duty lifting device attaches to mast and uses telescoping arm and gears to lift a dinghy or other heavy items to and from the boat (we used it to lift and launch our 10’ hard bottom inflatable dinghy stored on the deck)

Accommodations

General:  *Propane for stove: 2 ea 5-gallon/20lb tanks with fill indicators store in cockpit locker with overboard drain;   * Fresh water storage and capacity:  Three tanks store fresh water for a total of 100 gallons;   *Port and starboard stainless steel tanks are baffled, have clean-out access plates, are bonded to the ships grounding system and store 50 and 35 gallons of water respectively;   *Nauta flexible water tank stores 15 gallons of water;   *Tank Tender gauge shows water level of stainless steel tanks;   *Hot and cold pressure water system run by SureFlo pump;  *Force 10 stainless steel hot water tank provides 6 gallons of hot water heated by 120vac or engine, new in 2009;   *12V Refrigeration provided by holding plate driven by a Danfoss BD50F compressor that is fresh-water cooled from a tank mounted in the lazarette storage space; system installed in 2007;   *Dickinson “Arctic” diesel stove provides heat for the entire cabin and is gravity-fed by a 2-gal day tank so heater can run silently without any pump noise; a 12vdc pump allows diesel to be transferred from the main tank to the day tank;   *Air Circulation: Opening butterfly and V-berth hatches, 12 opening port holes and 4 dorade vents provide cooling air circulation for the entire cabin;  *Cabin can sleep 6 people:  1 in quarter berth, 2 in double bed made by dropping salon table, 1 on starboard settee, 2 in V-berth.

Galley:  *Two deep stainless steel galley sinks with hot and cold pressurized water faucets;   *Sink covers add more counter space;   *Cold water faucet has 0.5micron water filter with removable cartridge;   *Backup fresh water hand pump;   *Salt water hand pump for offshore use;    *Gimbaled 3-burner propane stove, grill and oven with propane valve switch, stove can be locked when gimballing not needed;   *12V refrigeration unit using Danfoss BD50 compressor cools the  galley’s ice box;   *Folding table extends galley work area but folds down when not in use;   *Galley storage: 6 drawers, large bin beside sink, 3 louvered lockers behind stove provide secure storage when sailing, corner locker, locker beneath stove, spice locker, under sink storage and food pantry (left of companionway ladder);   *Place settings for 4+ (plates, bowls, glasses, mugs, silverware),  *Cooking knives and misc cooking utensils,  *Miscellaneous cookware;   *Wall-mounted silverware and knife racks;   *Fire extinguisher mounted on aft bulkhead.

Nav Station/Quarter Berth:  *Furuno radar (see “Navigation”) on top of bookshelf,  *Large chart table;   *Swift 7x35 binoculars;   *Sony AM/FM radio and CD player;   *XM radio with marine antenna mounted on stern pulpit;  *ICOM IC-M402S VHF radio;   *3 ea dual-receptacle inverter outlets;   *3 ea 12V outlets;   *Nav Station storage: large book shelf area, large chart locker beneath chart locker, 4 drawers, locker underneath chart table, louvered locker next to control panel for storing Autohelm autopilot and anchor lantern, locker beneath control panel;   *Line storage rack in  quarter berth   *Rear quarter berth door provides access to cockpit lazarette where bosun chair and mast-hauling rig is stored,   *Teak fittings for dowels so quarter berth can be used as hanging locker for liveaboard;   *Quarter berth storage: rear quarter berth louvered locker, storage above quarter berth ceiling, storage beneath rear quarter berth cushion.

Salon:  *L-shaped port settee surrounds salon table that seats 4 comfortably for a meal;   *Table can be dropped to make a double bed using settee back cushions;   *Solid teak table extension sits upon salon table drawer with external support leg and extends meal seating to 8 using two folding chairs;   *Table extension and leg can be stored beneath the port settee;   *Washable port and starboard settee cushion covers  protect cushions; *Den Haan Trawler Lamp hangs from ceiling above salon table and provides light and heat for the salon;   *A second kerosene lamp  is mounted near the Dickinson stove;   *A 1-gal kerosene tank is stored in locker below the diesel day tank;   *Rotating fan over port side corner shelf circulates air in the salon;   *Dickinson Arctic diesel stove provides heat for the entire cabin (see “General” above);   *Fan above the Dickinson circulates heat from stove pipe throughout the cabin;   *Audio speakers are mounted in forward port and starboard louvered lockers;   *17” Philips LCD HDTV extends and rotates for salon viewing on articulated wall mount next to starboard settee (see photos);   *Panasonic DVD player and LCD TV situated for remote control from port or starboard settees;  *Opening butterfly hatch provides light and circulation;   *Salon storage:  two large bookshelves,  two small bookshelves; four louvered lockers, lockers behind settee back cushions, bin beside Dickinson heater, locker below diesel day tank.

V-berth:  *Queen-size V-berth with 4” mattress topper that includes 2” memory foam,    *Small cushioned seat bench for dressing;   *Bathroom door can be swung to close-off salon from V-berth area;   *Silva compass above V-berth for reading boat’s heading;   *Opening V-berth hatch provides light and circulation;   *V-berth storage:  Hanging locker, port and starboard louvered V-berth lockers, 3 large drawers, 1 small drawer, bench locker, large storage area beneath aft port V-berth cushions, storage underneath and beside small drawer, small locker below anchor locker, storage below forward V-berth cushions.

Head/Bathroom:  *Raritan PHII marine toilet with new pump assembly as of 2008;   *Hot and cold pressurized water faucet with switch for shower head;   *Stainless steel sink and marble sink counter;   *Brass water glass and soap dish;   *Large teak-framed mirror;   *Shower rail on ceiling for shower curtain;   *Shower water collects beneath grated floor where sump pump pumps water overboard preventing water from draining into bilge,   *Bathroom storage; 3 drawers, 3 lockers, under sink.

Notes:

*Information and specifications are believed to be correct but are not warranted or guaranteed.

*Four binders contain equipment specifications, user manuals, diagrams and other pertinent boat documentation.

*Spare parts lists are currently under construction

See Picasa Photos at: https://picasaweb.google.com/112961539744233101972/MinstrelOfSeattle?authuser=0&authkey=Gv1sRgCPm576Wgz6m21AE&feat=directlink